1965 C-172F SKYHAWK NORMAL PROCEDURES

AIRSPEEDS		MPH IAS

VS0				38

VS1				44

VR				60

VX				66

Short/Soft Final (40° flaps)		70

Normal Final (30° flaps)		75

VY				80

VMax Glide				80	(Glide Ratio = 1.5 SM/1,000’ AGL)

Final (0° flaps)			80

Base (20° flaps)			80

Past Abeam (10° flaps)		90

Cruise Climb			90

VFE				100

Downwind			100

VA				122

VNO				140

VNE				174

X Wind			KTS

Max Demonstrated		13

PREFLIGHT

Cabin

- Papers – A.R.O.W.

- Control Wheel Lock – REMOVE

- Mags – OFF

- Avionics – OFF

- Circuit Breakers – IN

- Trim – TAKEOFF

- Fuel Selector – CHECK EACH POSITION, SET BOTH

- Prop – CLEAR

- Master – ON

- Fuel Gauges – TRUE

- Flaps – EXTEND

- Lights – INTERIOR / EXTERIOR

- Pitot Heat – TEST

- Stall Indicator – TEST

- Master – OFF

- Fuel Strainer – PULL (4 sec)

- Baggage Door – LOCK

1965 C-172F SKYHAWK NORMAL PROCEDURES

Empennage

- Left Elevator – CHECK

- Tiedown – DISCONNECT

- Rudder – CHECK

- Rudder and Elevator Cables – CHECK

- Navigation Light – SECURE

- Trim Tab Alignment – CHECK (within ¼” of elevator)

- Right Elevator – CHECK

- Antennas – CHECK

Right Wing

- Flap – CHECK

- Aileron – CHECK

- Strobe/Navigation Lights – CHECK

- Tiedown – DISCONNECT

- Fuel Quantity – CHECK VISUALLY

- Fuel Filler Cap – SECURE

- Right Fuel Tank Sump – DRAIN

- Chock – REMOVE

- Tire – CHECK (24 PSI)

- Brake Assembly – CHECK

Nose

- Chock – REMOVE

- Nose Strut – 1-3” EXTENSION

- Nose Tire – CHECK (26 PSI)

- Exhaust System – CHECK

- Air Filter – CLEAR

- Engine Air Inlets – CLEAR

- Prop/Spinner – CHECK

- Oil Level – 7 QTS (under 3 HR flight), 8 QTS (extended flight)

- Oil Cap – SECURE

- Engine Compartment – CHECK

- Access Door – SECURE

- Static Port – CLEAR

Left Wing

- Chock – REMOVE

- Tire – CHECK (24 PSI)

- Brake Assembly – CHECK

- Left Fuel Tank Sump – DRAIN

- Fuel Quantity – CHECK VISUALLY

- Fuel Filler Cap – SECURE

- Pitot Tube – CLEAR

- Fuel Vent – CLEAR

- Tiedown – DISCONNECT

- Landing/Taxi Lights – CHECK

- Strobe/Navigation Lights – CHECK

- Aileron – CHECK

- Flap – CHECK

1965 C-172F SKYHAWK NORMAL PROCEDURES

START

- Hobbs Time – RECORDED

- Passenger Brief – COMPLETE

- Seat Track/Back – LOCK

- Seat Belt /Harness – FASTEN

- Brakes – SET

- Beacon – ON

- Navigation Lights – AS REQUIRED

- Mixture – RICH

- Throttle – OPEN 1/8”

- Carb Heat – OFF

- Prime – AS REQUIRED

- Prop – CLEAR

- Master – ON

- Mags – START

- Throttle – 1,000 RPM

- Ammeter - CHECK

- Oil Pressure – CHECK

- Mixture – LEAN

PRE-TAXI

- Flaps – UP

- Cabin Heat & Air – AS DESIRED

- Avionics – ON / SET

- Transponder – CODE / STANDBY

- Clock – SET

- Airspeed Indicator – CHECK

- Attitude Indicator – SET

- Heading Indicator – SET

- VSI – CHECK

- ATIS/AWOS – COPY

- Altimeter – SET / CHECK

- Taxi Light – AS REQUIRED

TAXI

- Brakes – TEST

- Attitude Indicator – CHECK

- Turn-and-Slip Indicator – CHECK

- Heading Indicator – CHECK

- Magnetic Compass – CHECK

1965 C-172F SKYHAWK NORMAL PROCEDURES

RUN-UP

- Brakes – SET

- Mixture – BEST POWER

- Primer – IN & LOCKED

- Throttle – 1,600 RPM

- Mags (R & L) – CHECK (125 RPM max drop, 75 RPM max differential)

- Carb Heat – TEST

- Vacuum – CHECK

- Ammeter – CHECK (0 to +2 needle widths)

- Oil Pressure – CHECK

- Oil Temperature – CHECK

- Throttle – IDLE CHECK

- Throttle – 1,000 RPM

- Mixture – LEAN

TAKEOFF

- Heading Indicator – SET

- Avionics – SET

- Engine Gauges – CHECK

- Fuel Quantity – CHECK

- Fuel Selector – BOTH

- Carb Heat – OFF

- Mixture – BEST POWER

- Trim – TAKEOFF

- Flaps – SET (0-10°)

- Flight Controls – FREE & CORRECT

- Doors/Window – LOCKED

- Takeoff Brief – COMPLETE

Taking Runway

(Lights, Camera, Action)

- Strobes – ON

- Landing Light – ON

- Transponder – ALTITUDE

- Mixture – BEST POWER

AFTER TAKEOFF

- Flaps – UP

- Engine Gauges – CHECK

- Landing Light – AS DESIRED

AIRSPEEDS		MPH IAS

VR				60

VX				66

VY				80

Cruise Climb			90

1965 C-172F SKYHAWK NORMAL PROCEDURES

CRUISE

- Landing Light – OFF

- Throttle – SET

- Mixture – LEAN

- Engine Gauges – CHECK

- Heading Indicator – SET

- Fuel Selector – BALANCE AS REQUIRED

FUEL MANAGEMENT ABOVE 5,000’ MSL

- When reaching cruise altitude above 5,000’ MSL, promptly switch fuel selector from BOTH to either LEFT or RIGHT and then to opposite tank to ensure both tanks are feeding.

- During cruise, use LEFT and RIGHT tank (not  BOTH) as required to manage fuel balance.

APPROACH

- Mixture – RICHEN AS REQUIRED

- Fuel Selector – BOTH (below 5,000’ MSL)

- Carb Heat – AS REQUIRED

- ATIS/AWOS – COPY

- Altimeter – SET

- Avionics – SET

- Heading Indicator – SET

- Engine Gauges – CHECK

- Brakes – PEDAL TEST

- Seatbelt/Harness – FASTENED

- Landing Light – ON

LANDING

- Fuel Selector – BOTH

- Mixture – BEST POWER

- Carb Heat – ON

- Flaps – AS DESIRED

AIRSPEEDS		MPH IAS

Downwind			100

Past Abeam (10° flaps)		90

Base (20° flaps)			80

Final (0° flaps)			80

Normal Final (30° flaps)		75

Short/Soft Final (40° flaps)		70

1965 C-172F SKYHAWK NORMAL PROCEDURES

AFTER LANDING

- Carb Heat – OFF

- Mixture – LEAN

- Flaps – UP

- Strobes – OFF

- Landing Light – OFF

- Taxi Light – AS REQUIRED

- Pitot Heat – OFF

- Transponder – STANDBY

SHUTDOWN

- Avionics – OFF

- Fuel Selector – RIGHT

- Throttle – 1,000 RPM

- Mags – GROUNDING CHECK

- Mixture – FULL LEAN

- Prop – CHECK STOPPED

- Mags – OFF & KEY ON DASH

- Master – OFF

- Lights – OFF

SECURING

- Hobbs/Tach Time – RECORDED

- Control Wheel Lock – INSTALL

- Chocks – INSTALL

- Tiedowns – SECURE

- Baggage Door – CLOSED

- Cabin Doors/Window – CLOSED

## 1965 C-172F SKYHAWK ABNORMAL PROCEDURES

FLOODED ENGINE

- Mixture – FULL LEAN

- Throttle – FULL OPEN

- Mags – START (hold for several revolutions)

- Repeat START checklist beginning on step 10 (do not reprime).

GROUND BURNOUT

- Mixture – RICH

- Throttle – 2,000 RPM

- Mixture – LEAN TO 50 RPM DROP BELOW BEST POWER

After 1 minute:

- Mixture – RICH

- Mags – RECHECK

RADIO FAILURE

- Volume – CHECK

- Headset Connection  – CHECK

- Audio Controls – CHECK

- Circuit Breakers – CHECK

- Speaker – ON

- Hand-Held Microphone – CHECK

- Frequency – SWITCH

If IFR (or VFR in Class B, C, D airspace):

- Transponder – 7600

If failure occurs in VFR conditions, or VFR conditions are encountered after failure, continue flight under VFR and land as soon as practicable.

VFR Landing at a Controlled Field:

- Remain outside or above Class D airspace until traffic flow determined.

- Transmit intentions “in the blind” advising tower of aircraft type, position, altitude, and intention to land.

- Enter pattern, report position “in the blind”, and watch for light signals from tower.

If failure occurs in IFR conditions, continue flight according to the following:

Route to Fly (in order): (AVE. F)

A – Assigned

V – Vectored

E – Expected

F – Filed

CONTINUED NEXT PAGE

## 1965 C-172F SKYHAWK ABNORMAL PROCEDURES

Altitude (highest of for the route segment being flown): (MEA)

M – Minimum IFR Altitude

E – Expected as Advised by ATC

A – Assigned by ATC

Descent for Approach:

From enroute altitude upon reaching the IAF but not before:

- Expect-further-clearance time (if given)

- ETA as calculated from filed ETE

| ATC LIGHT SIGNALS |  |  |
| --- | --- | --- |
| SIGNAL | ON DECK | AIRBORNE |
| Steady Green | Cleared for Takeoff | Cleared to Land |
| Flashing Green | Cleared for Taxi | Return for Landing (to be followed by steady green) |
| Steady Red | STOP | Give way to other aircraft and continue circling |
| Flashing Red | Taxi clear of runway in use | Airport Unsafe – DO NOT LAND |
| Flashing White | Return to starting point on airport | Not Applicable |
| Alternating Red and Green | Exercise extreme caution | Exercise extreme caution |
| Acknowledge all light signals by flashing landing light and/or rocking wings. |  |  |

## 1965 C-172F SKYHAWK EMERGENCY PROCEDURES

ABORT

- Throttle – IDLE

- Brakes – AS REQUIRED

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed – 80 MPH (flaps UP)

- Mixture – FULL LEAN

- Fuel Selector – OFF

- Flaps – DOWN (70 MPH glide)

- Master & Mags – OFF

COMPLETE ENGINE FAILURE DURING FLIGHT

- Airspeed – 80 MPH (flaps UP)	(Glide Ratio = 1.5 SM/1,000’ AGL)

If Airstart Warranted:

- Carb Heat – ON

- Mixture – RICH

- Fuel Selector – BOTH

- Primer – IN & LOCKED

- Mags – CHECK ALL

- Master – ON

FORCED LANDING

- Airspeed – 80 MPH (flaps UP)

- Squawk –7700

- Emergency – DECLARE

- ELT – ON

- Mixture – FULL LEAN

- Fuel Selector – OFF

- Seatbelt/Harness – FASTENED

- Flaps – AS REQUIRED (70 MPH with flaps full)

- Master & Mags – OFF

- Doors – UNLATCH

## 1965 C-172F SKYHAWK EMERGENCY PROCEDURES

ENGINE ROUGHNESS

# If over 5,000’ MSL:

- Vapor In Fuel Tank – EXECUTE

# If 5,000’ MSL or below, or Vapor In Fuel Tank procedure doesn’t correct:

- Carb Heat – ON

If roughness continues after 1 min:

- Carb Heat – OFF

- Mixture – ADJUST FOR MAX SMOOTHNESS

- Fuel Selector – BOTH

- Primer – IN & LOCKED

- Engine Gauges – CHECK

- Mags – L then R then BOTH

- Mags – SMOOTHEST SETTING

- Power – REDUCE

- Mixture – FULL RICH

- LAND AT NEAREST AIRPORT

VAPOR IN FUEL TANK (above 5,000 MSL)

During Operation on a Single Tank:

- Fuel Selector – SWITCH TO OPPOSITE TANK

During Operation on Both Tanks:

- Fuel Selector – SWITCH TO A SINGLE TANK FOR 60 SECONDS

- Fuel Selector – SWITCH TO OPPOSITE TANK

HIGH OIL TEMPERATURE

- Power – REDUCE

- Mixture – RICH

- Airspeed – INCREASE

If high temp persists:

- LAND AT NEAREST AIRPORT

LOSS OF OIL PRESSURE

- Oil Temp – CHECK

- Power – REDUCE

If oil temp shows rise, engine failure imminent:

- Forced Landing – EXECUTE

If oil temp does not show rise:

- LAND AT NEAREST AIRPORT

## 1965 C-172F SKYHAWK EMERGENCY PROCEDURES

ENGINE FIRE ON GROUND

- Mixture – FULL LEAN

- Fuel Selector – OFF

- Mags – OFF

- Master – OFF

ENGINE FIRE IN FLIGHT

- Mixture – FULL LEAN

- Fuel Selector – OFF

- Master & Mags – OFF

- Cabin Heat & Air – OFF

- Airspeed – 105 MPH

- Forced Landing – EXECUTE

ELECTRICAL FIRE

- Master – OFF

WING FIRE

- Landing/Taxi Lights – OFF

- Navigation Lights – OFF

- Strobe Lights – OFF

- Pitot Heat – OFF

- Sideslip – AS REQUIRED

- Forced Landing – EXECUTE

SMOKE AND FUMES IN COCKPIT

- Cabin Heat Knob – IN

- Cabin Air Knob – IN

- Upper Air Vents – OPEN

- Pilot’s Window – AS REQUIRED (below 100 MPH)

## 1965 C-172F SKYHAWK EMERGENCY PROCEDURES

HIGH AMMETER READING

(more than 2 needle widths positive deflection)

- Master – OFF

NEGATIVE AMMETER READING

- Electrical Load – REDUCE

VACUUM SYSTEM FAILURE

If VMC:

- Attitude Indicator & Heading Indicator – COVER

- Maintain VMC

If IMC:

- Attitude Indicator & Heading Indicator – COVER

- Fly using Turn-and-Slip Indicator, Altimeter, and Airspeed Indicator

- Declare EMERGENCY

INOPERATIVE TRIM

- Flaps – AS REQUIRED

- Power – AS REQUIRED

FLAP MALFUNCTION

- Circuit Breaker – CHECK / RESET

If breaker pops again:

- Flaps – LEAVE WHERE SET

STRUCTURAL ICING

- Pitot Heat – ON

- Carb Heat – ON

- Throttle – MAX RPM

- Mixture – BEST POWER

- Cabin Heat – MAX DEFROST

- EXIT ICING CONDITIONS

If landing with structural icing:

- Flaps – LEAVE UP

## 1965 C-172F SKYHAWK EMERGENCY PROCEDURES

DOOR OR WINDOW OPENING IN FLIGHT

- FLY THE AIRPLANE

If Window:

- Window – CLOSE / LOCK

# If Door:

- LAND AS SOON AS PRACTICABLE

LANDING WITH A FLAT MAIN TIRE

  - Approach – NORMAL

  - Flaps – 30°

  - Touchdown – GOOD TIRE FIRST

  - Hold airplane off flat tire as long as possible

  - Maintain directional control using good-tire brake

LANDING WITH A FLAT NOSE TIRE

- Approach – NORMAL

- Flaps – AS REQUIRED

- Touchdown – ON MAINS

- Hold nosewheel off ground as long as possible

When nosewheel touches down – FULL UP ELEVATOR

SPIN RECOVERY				Memory Aid: PARE

- Power – Idle					(Power)

- Ailerons – Neutral					(Ailerons)

- Rudder – Full opposite rotation direction		(Rudder)

- Yoke – Full forward with neutral ailerons		(Elevator)

- Rudder – Neutral when rotation stops

- Yoke – Pull smoothly from dive

## C-172F Abbreviated Checklist, 23 Nov 03

# INITIAL

Wx & Density Alt

Weight & Balance

Performance Req

Flight Plan - File

Papers - A.R.O.W.

Control Lock

Fuel - Both

Master - On

Flaps - Extend

Pitot Heat - Test

Lights - Int/Ext

Stall Indicator - Test

Fuel Gauges - True

Master - Off

Fuel Strainer

# EXTERIOR

Fuel Quantity

Fuel Quality

Caps/Drains/Vents

Engine/Oil

Prop/Air Intake

Exhaust System

Surfaces & Controls

Pitot & Static Ports

Gear/Tires/Brakes

Antennas

Ties/Chocks

Baggage Door

Final Walk Around

# INTERIOR

Passenger Brief

Hobbs Time

Circuit Breakers

Brakes - Pedal Test

# START

Seat Track/Back - Lock

Avionics - Off

Carb Heat - Off

Beacon - On

Nav Lights - As Req

Mixture - Full Rich

Throttle - Slight

Prime - As Req

Brakes

Prop - Clear

Master - On

Mags - Start

1,000 RPM

Ammeter

Oil Pressure

Mixture - Lean

# PRE-TAXI/TAXI

Seat Belts/Harness

Flaps - Up

Heat/Vent

Avionics - On/Set

Transponder - Stby

ATIS/AWOS

Altimeter - Set

Taxi Light - As Req

Brakes - Test

Attitude Indic - Test

Turn & Slip - Test

H.I./Compass - Test

# RUN-UP

Brakes - Set

Fuel - Both

Trim - Takeoff

Flight Controls

Instruments

Mixture - Best Power

Primer - In & Locked

1600 RPM

Mags (R&L) - Test

Carb Heat - Test

Vacuum

Amps

Oil Pressure

Idle - Check Closed

Mixture - Lean

# TAKEOFF

Flaps - 0-10°

Carb Heat - Off

Pitot Heat - As Req

H.I. to Compass

Doors/Window

Takeoff Brief

# TAKING RUNWAY

(Lights, Camera, Action)

Strobes - On

Landing Light - On

Transponder - Altitude

Mixture - Best Power

# CLIMB

80-90 MPH

Flaps - Up

Power

Mixture

Instruments

Landing Light - Off

# CRUISE

Power

Mixture

Instruments

Fuel Selector - As Req

H.I. To Compass

# APPROACH

Mixture - Richen

Fuel - Both

Carb Heat - As Req

ATIS/AWOS

Altimeter - Set

Instruments

H.I. To Compass

Landing Light - On

Seatbelt/Harness

# LANDING

Fuel - Both

Mixture - Best Power

Carb Heat - On

Flaps - As Req

# AFTER LANDING

Flaps - Up

Carb Heat - Off

Mixture - Lean

Strobes - Off

Landing Light - Off

Taxi Light - As Req

Pitot Heat - Off

Trim - Takeoff

Transponder - Stby

# SHUTDOWN

Avionics - Off

Fuel - Right

Mags - Ground Check

Mixture - Full Lean

Mags - Off

Master - Off

Lights - Off

# SECURING

Hobbs/Tach Time

Control Lock

Chocks

Tie Downs

Baggage Door

Cabin Doors/Window

# AIRSPEEDS MPH IAS

VS0	38

VS1	44

VR	60

VX	66

Short/Soft Final (40)	70

Normal Final (30)	75

VY	80

Vmax Glide	80

Final (0°)	80

Base (20°)	80

Past Abeam (10°)	90

Cruise Climb	90

VFE	100

Downwind	100

VA	122

VNO	140

VNE	174

# DATA

Engine:	Continental

O-300-D

145 HP

Max T.O. Weight:	2300 #

Max Bag Area:	120 #

Fuel:	100LL (Blue)

Usable Fuel:	36 Gal

Oil Capacity:	8 Qts

(Min 6)

Electrical:	12-14 V

60 Amp

Nose Tire:	26 PSI

Main Tire:	24 PSI