Normal Checklist Cessna 150

BEFORE STARTING ENGINE

- Exterior Preflight -- COMPLETE.

- Tach/Hobbs Meter -- RECORD

- Seats, Belts, Harnesses -- ADJUST & LOCKED.

- Fuel Shutoff Valve -- ON.

- Radios & Electrical Equipment -- OFF.

- Brakes -- TEST & SET.

STARTING ENGINE

- Mixture -- RICH.

- Carburetor Heat -- COLD.

- Master Switch –- ON.

- Prime –- AS REQUIRED.

- Throttle –- OPEN ¼ INCH.

- Propeller Area -– CLEAR.

- Ignition Switch –- START. (release when engine starts)

- Oil Pressure –- CHECK.

BEFORE TAKEOFF

- Cabin Doors –- LATCHED.

- Flight Controls -– FREE & CORRECT.

- Elevator Trim –- TAKEOFF POSITION.

- Fuel Shutoff Valve –- ON.

- Brakes -- SET.

- Throttle -– 1700 RPM.

- Magnetos –- CHECK.

- Carburetor Heat –- CHECK FOR RPM DROP.

- Engine Instruments and Ammeter –- CHECK.

- Suction Gauge –- CHECK.

7.  Flight Instruments and Radio -- SET.

8.  Throttle Friction Lock -– ADJUST.

9.  Wing Flaps -– AS REQUIRED.

TAKEOFF

- Normal Takeoff

- Wing Flaps -– UP (0).

- Carburetor Heat –- COLD.

- Throttle –- FULL OPEN.

- Elevator Control –- LIFT NOSE WHEEL.

- Climb Speed -– 70-80 MPH.

- Maximum Performance Takeoff

- Wing Flaps –- UP (0).

- Carburetor Heat -– COLD.

- Brakes -– HOLD.

- Throttle –- FULL OPEN.

- Brakes –- RELEASE.

- Elevator Control –- SLIGHTLY TAIL LOW.

- Climb Speed -– 70 MPH (with obstacles ahead).

ENROUTE CLIMB

- Airspeed -– 75 – 85 MPH.

- Throttle -– FULL OPEN.

- Mixture –- RICH. (Unless engine is rough).

CRUISE

- Power –- 2000 to 2750. (No more than 75%).

- Elevator Trim –- ADJUST.

- Mixture -– LEAN.

BEFORE LANDING

- Mixture -– RICH

- Carburetor Heat –- ON (apply before closing throttle).

- Airspeed –- 70-80 MPH (flaps up).

- Wing Flaps –- AS DESIRED (below 100 MPH)

- Airspeed -– 60-70 MPH (flaps down).

BALKED LANDING

- Throttle –- FULL OPEN.

- Carburetor Heat –- COLD.

- Wing Flaps –- RETRACT TO 20.

- Airspeed –- 65 MPH

- Wing Flaps –- RETRACT  (slowly).

NORMAL LANDING

- Touchdown –- MAIN WHEELS FIRST.

- Landing Roll -– LOWER NOSE WHEEL GENTLY.

- Braking -– MINIMUM REQUIRED.

AFTER LANDING

- Wing Flaps –- UP.

- Carburetor Heat –- COLD.

- Unnecessary Equipment -– OFF.

SECURING AIRCRAFT

- Parking Brake -– SET

- Radios, Electrical Equipment –- OFF

- Mixture –- IDLE CUT-OFF

- Magnetos -– OFF

- Master Switch –- OFF.

- Control Lock –- INSTALL.

- Tach/Hobbs Meter –- RECORD.

C150G/C150L Airspeeds (MCAS)

Vr………………………...………………55

Vx………………………..………...…..70

Vy……………………………….…..…..78

Va…………………...….……………..109

Vs1……………….…..…..…………...55

Vso…………………….…………..…..48

Vfe…………………………..………..100

Vno…………………………..…..…..120

Vne……………………..……….…..162

Glide ………………………….…..…..70

XW demo…………..…….……..…..15